eSim One-Click Simulation Bridge

P-BRIDGE Plugin for KiCad 8.0

User Manual & Technical Guide

FOSSEE Semester Long Internship Spring 2026

Task 6: KiCad Plugin Development

IIT Bombay

Version 1.0.0 | March 2026

# 1. Introduction

The eSim One-Click Simulation Bridge (P-BRIDGE) is a KiCad 8.0 plugin developed as part of the FOSSEE Semester Long Internship Spring 2026 at IIT Bombay. The plugin bridges the gap between KiCad schematic design and eSim circuit simulation, enabling engineers and students to simulate their circuits with a single click.

## 1.1 Problem Statement

Without this plugin, simulating a KiCad schematic in eSim requires seven manual steps:

- Export netlist from KiCad using kicad-cli

- Open eSim and create/open a project

- Copy the netlist file to the eSim workspace

- Convert KiCad netlist to SPICE format

- Configure the analysis type and parameters

- Write the .control block manually

- Run the simulation and plot results

The P-BRIDGE plugin automates all of these steps into a single button click.

## 1.2 Solution

The plugin provides a graphical dialog within KiCad for selecting analysis type and parameters, then automatically handles all conversion, file writing, and eSim launching steps. The user only needs to click 'Simulate' and 'Plot' in eSim after the plugin completes.

## 1.3 Key Features

- One-click workflow from KiCad schematic to eSim simulation

- Supports Transient, AC, DC Sweep, and Operating Point analysis

- Automatic SPICE netlist generation from KiCad components

- Supports resistors, capacitors, inductors, voltage sources, diodes, BJTs, MOSFETs

- AC analysis with Lin/Dec/Oct scale selection

- Transient analysis with configurable start, step, and stop time

- DC sweep with configurable source, range, and step

- Operating Point analysis with direct result display

- Preflight checks for system requirements

- Detailed logging for debugging

# 2. System Requirements

## 2.1 Software Requirements

| Software | Version | Notes |
| --- | --- | --- |
| Ubuntu Linux | 20.04 / 22.04 / 24.04 | Tested on 24.04 |
| KiCad | 8.0 | Must include kicad-cli |
| eSim | 2.5 | Installed at ~/Downloads/eSim-2.5/ |
| Python | 3.10+ | System Python |
| ngspice | Any recent | Included with eSim 2.5 |
| Pillow (PIL) | Any | For icon creation only |

## 2.2 Hardware Requirements

- RAM: 4 GB minimum, 8 GB recommended

- Storage: 2 GB free space for eSim and simulation data

- Display: 1280x720 minimum resolution

## 2.3 Verifying Requirements

Run the following commands to verify your system meets all requirements:

Check KiCad version:

kicad-cli --version

Check eSim installation:

ls ~/Downloads/eSim-2.5/src/frontEnd/Application.py

Check ngspice:

ngspice --version

Check Python:

python3 --version

# 3. Installation

## 3.1 Download the Plugin

Clone or download the plugin from the repository:

git clone https://github.com/YOUR_USERNAME/esim-bridge-plugin.git

## 3.2 Install Plugin Files

Copy the entire plugin folder to the KiCad plugins directory — just one command:

cp -r esim-bridge-plugin/esim_bridge ~/.local/share/kicad/8.0/scripting/plugins/

No manual folder creation needed. The cp -r command copies the entire esim_bridge folder including all files.

## 3.3 Verify Installation

Confirm the files are in place:

ls ~/.local/share/kicad/8.0/scripting/plugins/esim_bridge/

You should see: esim_bridge.py, icon.png, __init__.py

## 3.4 Restart KiCad

Close and reopen KiCad completely. The plugin will be loaded automatically on startup.

## 3.5 Confirm Plugin is Loaded

Open KiCad PCB Editor. You should see the eSim Bridge icon in the toolbar. Alternatively, go to Tools → External Plugins → eSim Simulation Bridge.

## 3.6 Plugin File Structure

After installation, the plugin directory should contain:

~/.local/share/kicad/8.0/scripting/plugins/esim_bridge/

esim_bridge.py      (main plugin — 49 KB)

icon.png            (toolbar icon — 4 KB)

__init__.py         (package marker)

# 4. Using the Plugin

## 4.1 Preparing Your Schematic

Before using the plugin, ensure your KiCad schematic is properly set up for simulation:

### 4.1.1 Adding Components

- Place resistors (R), capacitors (C), inductors (L) with their values

- Add a voltage source — use VSIN for AC/Transient, or a standard V for DC

- Connect all components with wires

- Add a GND power symbol at the bottom of your circuit

### 4.1.2 Configuring the Voltage Source

For VSIN (sine wave source), double-click V1 and set simulation model parameters:

| Parameter | KiCad Field | Example | Description |
| --- | --- | --- | --- |
| dc | Sim.Params | dc=0 | DC offset voltage |
| ampl | Sim.Params | ampl=1 | Sine wave amplitude (V) |
| f | Sim.Params | f=1k | Frequency (Hz) |
| ac | Sim.Params | ac=1 | AC magnitude for .ac analysis |

### 4.1.3 Adding Net Labels

Add net labels (press L in schematic editor) to important nodes for easier identification in simulation results. For example, label the midpoint of a voltage divider as 'vmid'.

## 4.2 Running the Plugin

- Open your schematic in KiCad Schematic Editor

- Click the eSim Bridge icon in the KiCad PCB Editor toolbar

- The Analysis Configuration Dialog will appear

- Select your analysis type

- Enter the analysis parameters

- Click 'Simulate in eSim →'

- The plugin will export, convert, and launch eSim

- A success message will confirm eSim has launched

## 4.3 Analysis Configuration Dialog

### 4.3.1 Transient Analysis

Simulates circuit behavior over time. Shows voltage/current vs time waveforms.

| Parameter | Default | Description |
| --- | --- | --- |
| Start Time | 0 | Simulation start time (e.g., 0, 1ms) |
| Step Time | 1us | Time resolution (e.g., 1us, 100ns) |
| Stop Time | 10ms | Simulation end time (e.g., 10ms, 1s) |

Generated SPICE command: .tran <step> <stop> <start>

Example: .tran 1us 10ms 0

### 4.3.2 AC Analysis

Frequency sweep analysis showing magnitude/phase vs frequency.

| Parameter | Default | Description |
| --- | --- | --- |
| Scale | dec | dec=decades, lin=linear, oct=octaves |
| Start Frequency | 1Hz | Start of frequency sweep |
| Stop Frequency | 1MEGHz | End of frequency sweep |
| No. of Points | 100 | Points per decade/octave or total |

Generated SPICE command: .ac <scale> <points> <fstart> <fstop>

Example: .ac dec 100 1Hz 1MEGHz

### 4.3.3 DC Sweep

Sweeps a DC source voltage and plots circuit response.

| Parameter | Default | Description |
| --- | --- | --- |
| Source Name | V1 | Name of the DC source to sweep |
| Start Value | 0 | Starting voltage (V) |
| Stop Value | 5 | Ending voltage (V) |
| Step | 0.1 | Voltage increment per step (V) |

Generated SPICE command: .dc <source> <start> <stop> <step>

Example: .dc V1 0 5 0.1

### 4.3.4 Operating Point

Calculates DC bias point of the circuit. No parameters required. Results are displayed directly in a message box showing node voltages. No waveform graph is produced.

Generated SPICE command: .op

## 4.4 In eSim After Launch

- eSim opens automatically

- Double-click 'esim_bridge_project' in the left project tree

- Click the 'Simulate' button

- Click 'Plot' to view the waveform graph

# 5. Terminal Command Reference

## 5.1 Viewing Generated Files

### View the SPICE netlist (.cir)

cat ~/eSim-Workspace/esim_bridge_project/esim_bridge_project.cir

### View the SPICE with control block (.cir.out)

cat ~/eSim-Workspace/esim_bridge_project/esim_bridge_project.cir.out

### View the analysis configuration

cat ~/eSim-Workspace/esim_bridge_project/analysis

## 5.2 Verifying Simulation Data

### Check voltage plot data (first 10 lines)

head -10 ~/eSim-Workspace/esim_bridge_project/plot_data_v.txt

### Check current plot data

head -10 ~/eSim-Workspace/esim_bridge_project/plot_data_i.txt

### Count data points

wc -l ~/eSim-Workspace/esim_bridge_project/plot_data_v.txt

## 5.3 Manual ngspice Simulation

### Run simulation in batch mode

ngspice -b ~/eSim-Workspace/esim_bridge_project/esim_bridge_project.cir.out

### Run simulation interactively

ngspice ~/eSim-Workspace/esim_bridge_project/esim_bridge_project.cir.out

### Run with output to file

ngspice -b esim_bridge_project.cir.out > sim_output.txt 2>&1

## 5.4 Netlist Format Verification

### Check KiCad netlist export format

kicad-cli sch export netlist --output /tmp/test.net --format kicadsexpr your_schematic.kicad_sch

### View the exported netlist

cat /tmp/test.net | head -50

## 5.5 Plugin Debugging

### View plugin log

cat ~/.local/share/kicad/esim_bridge.log

### Follow log in real time

tail -f ~/.local/share/kicad/esim_bridge.log

### Check if plugin is installed

ls -la ~/.local/share/kicad/8.0/scripting/plugins/esim_bridge/

### Manually delete stale .raw file

rm -f ~/eSim-Workspace/esim_bridge_project/esim_bridge_project.raw

# 6. SPICE Netlist Format Reference

## 6.1 Basic SPICE Syntax

The plugin generates standard ngspice-compatible SPICE netlists. Here is the format for each component type:

| Component | SPICE Format | Example |
| --- | --- | --- |
| Resistor | R<name> <n+> <n-> <value> | R1 vin vmid 10k |
| Capacitor | C<name> <n+> <n-> <value> | C1 vmid 0 100nF |
| Inductor | L<name> <n+> <n-> <value> | L1 n1 n2 1mH |
| DC Source | V<name> <n+> <n-> DC <value> | V1 vin 0 DC 5 |
| Sine Source | V<name> <n+> <n-> AC <ac> SIN(<dc> <ampl> <f>) | V1 vin 0 AC 1 SIN(0 1 1k) |
| Pulse Source | V<name> <n+> <n-> PULSE(<v1> <v2> <td> <tr> <tf> <pw> <per>) | V1 n 0 PULSE(0 5 0 1n 1n 5m 10m) |
| Diode | D<name> <anode> <cathode> <model> | D1 n1 n2 D1N4148 |
| NPN BJT | Q<name> <c> <b> <e> <model> | Q1 n1 n2 n3 BC547 |
| MOSFET | M<name> <d> <g> <s> <b> <model> | M1 d g s b NMOS |

## 6.2 Control Block Format

The plugin appends a .control block to the SPICE file for simulation control:

* Control Statements

.control

run

print allv

print alli

print allv > plot_data_v.txt

print alli > plot_data_i.txt

.endc

.end

## 6.3 Example Complete SPICE File

Voltage divider circuit — Transient Analysis:

* eSim Bridge Plugin - Auto-generated SPICE file

* Source: /tmp/esim_bridge_netlist.net

* Components

R1 net_r1_pad1 vmid 10k

R2 vmid 0 10k

V1 net_r1_pad1 0 AC 1 SIN(0 1 1k)

* Simulation Analysis

.tran 1us 10ms 0

* Output

.print tran v(net_r1_pad1)

.print tran v(vmid)

.probe v(*)

* Control Statements

.control

run

print allv > plot_data_v.txt

print alli > plot_data_i.txt

.endc

.end

# 7. Troubleshooting

## 7.1 Common Issues and Solutions

| Error | Cause | Solution |
| --- | --- | --- |
| Plugin not visible in toolbar | KiCad not restarted after install | Restart KiCad completely |
| No schematic found | PCB editor open, not schematic | Run from PCB editor — plugin finds .kicad_sch automatically |
| kicad-cli not found | KiCad 8.0 not properly installed | Reinstall KiCad 8.0, verify with: kicad-cli --version |
| eSim not found warning | eSim not at expected path | Verify ~/Downloads/eSim-2.5/ exists |
| Data could not be plotted | analysis file format wrong | Check: cat ~/eSim-Workspace/esim_bridge_project/analysis |
| UTF-8 syntax error in terminal | Stale .raw binary file | Benign error, plugin auto-deletes .raw file on next run |
| ZeroDivisionError in eSim | analysis file empty or wrong | Re-run plugin, verify analysis file is written correctly |
| Please select project first | eSim loaded but project not selected | Double-click project name in eSim left panel |

## 7.2 Diagnostic Commands

Check if all project files exist:

ls -la ~/eSim-Workspace/esim_bridge_project/

Verify analysis file content:

cat ~/eSim-Workspace/esim_bridge_project/analysis

Test ngspice directly:

ngspice -b ~/eSim-Workspace/esim_bridge_project/esim_bridge_project.cir.out

Check plugin log for errors:

cat ~/.local/share/kicad/esim_bridge.log

## 7.3 Known Limitations

- Operating Point (.op) analysis: eSim 2.5 does not support graphical plotting of OP results. Node voltages are shown in a message box.

- UTF-8 terminal error: A benign error appears in the ngspice terminal when eSim opens after a previous simulation. Does not affect results.

- Auto project load: eSim 2.5 requires manually selecting the project in the project tree.

- Linux only: Plugin is designed for Ubuntu Linux with eSim 2.5.

- Single project: All simulations use the same project folder esim_bridge_project.

# 8. Developer Reference

## 8.1 Plugin Architecture

| Class | Purpose |
| --- | --- |
| SPICEConverter | Parses KiCad netlists and converts to SPICE format |
| AnalysisConfigDialog | wx.Dialog for selecting analysis type and parameters |
| ESimLauncher | Finds and launches eSim application |
| PreflightChecker | Validates system requirements before simulation |
| SimulationReadyDialog | Shows SPICE preview before eSim launch |
| ESimBridgePlugin | Main KiCad ActionPlugin class — coordinates workflow |

## 8.2 Plugin Workflow

- User clicks plugin → Run() called

- Stale .raw file deleted

- AnalysisConfigDialog shown — user selects analysis type/params

- Schematic path detected from open PCB project

- PreflightChecker validates kicad-cli, eSim, /tmp write access

- kicad-cli exports netlist to /tmp/esim_bridge_netlist.net

- SPICEConverter parses netlist and generates SPICE .cir file

- SPICE content written to ~/eSim-Workspace/esim_bridge_project/

- analysis file written with correct .tran/.ac/.dc/.op command

- SimulationReadyDialog shows preview — user confirms launch

- Stale .raw file deleted again

- eSim launched via subprocess.Popen

- Success message shown with instructions

## 8.3 Key File Paths

| File | Purpose |
| --- | --- |
| ~/.local/share/kicad/8.0/scripting/plugins/esim_bridge/esim_bridge.py | Main plugin code |
| ~/.local/share/kicad/esim_bridge.log | Plugin log file |
| /tmp/esim_bridge_netlist.net | Temporary KiCad netlist export |
| /tmp/esim_bridge_simulation.cir | Temporary SPICE file |
| ~/eSim-Workspace/esim_bridge_project/esim_bridge_project.cir.out | Final SPICE with control block |
| ~/eSim-Workspace/esim_bridge_project/analysis | Analysis type for eSim plotter |
| ~/eSim-Workspace/esim_bridge_project/plot_data_v.txt | Voltage simulation results |
| ~/eSim-Workspace/esim_bridge_project/plot_data_i.txt | Current simulation results |

# 9. Testing Checklist

## 9.1 Installation Tests

- Plugin appears in KiCad toolbar

- Plugin shows in Tools → External Plugins menu

- Clicking button shows Analysis Configuration Dialog

- Cancel button closes dialog without action

## 9.2 Analysis Type Tests

- Transient: generates .tran command with correct step/stop/start

- AC: generates .ac command with correct scale/points/fstart/fstop

- DC: generates .dc command with correct source/start/stop/step

- OP: shows node voltages in message box, does not crash eSim

## 9.3 SPICE Conversion Tests

- Resistors: R1 node1 node2 10k

- Capacitors: C1 node1 node2 100nF

- VSIN source: V1 n+ n- AC 1 SIN(0 1 1k)

- Ground nets become 0 in SPICE

- Net names with special characters are cleaned

- .end always present at end of file

## 9.4 Simulation Output Tests

- plot_data_v.txt has correct Index/time/node headers

- plot_data_i.txt has correct Index/time/alli headers

- analysis file matches selected analysis type

- Graph appears in eSim after Simulate → Plot

## 9.5 Error Handling Tests

- Shows error if no schematic open

- Shows warning if eSim not installed

- Shows error if kicad-cli not found

- Log file created at ~/.local/share/kicad/esim_bridge.log